DIGITAL  HAPPENINGS  #17
(December 2013)
By W0NAC (“Matt”)

The Christmas season has been a very busy one for the Matthew family and I’m sure for your families too. Having many of our children, grandchildren, and our 2 year old great granddaughter with us over Christmas has been super nice, but carving out time to write this issue of Digital Happenings has been tough. As a result, this month’s article will again be a bit shorter than usual (maybe this is a good trend?). My hope is that all of you had a meaningful and merry Christmas and that you are having a great 2014 new year so far! Now on to the main subject for this month’s article….

JT65 & JT9 Modes (Re-visited)

Even though I have discussed both JT65 and JT9 modes in past articles, there have been changes and updates (especially in software) that affect both modes.  Because of these new changes, I would like to re-visit both of these modes this month.

I have come to think of JT65 and JT9 modes as “hidden treasures” for county hunting. I believe this is true for several reasons. First, they are unbelievably popular not only in the US, but all over the world. One can find stations to work using these modes when every band appears to be dead!

Second, they are the absolute best modes you can use for working weak signals on HF.  This weak signal capability allows one to use low power and less than optimum antennas (like most mobile antennas and attic antennas) and still make reliable contacts.

If you give them a try, I think you will find JT65 and JT9 to be very helpful for working new counties for your USACA or Nth time as well as adding to your USA – Digital Award and 5 – Mode Award totals.  Also, for those county hunters that like to work DX, the JT65 and JT9 modes can help you reach your DXCC goals.

JT65 HF Mode General Description – The JT65-HF mode is based on JT65A code from the WSJT Project (headed by Joe Taylor - K1JT), but has been specially adapted for use on HF by Joe Large - W6CQZ. JT65 is a tightly structured protocol that sends a very limited set of short structured messages or a small ‘free text’ message limited to a maximum of 13 characters. It is not a free form data transmission protocol like most other modes, but each transmitted ‘frame’ is of a fixed length, starting and ending at a predefined time. In any one minute frame you can send any one of the structured messages, 13 characters of text or any of the 3 shorthand messages. Nothing more, nothing less.  Each new transmission starts precisely 1 second into a new minute and ends ~47 seconds later when a decoding interval starts for anyone listening to your signal. This structure, its timing, modulation method, and heavy Forward Error Correction (FEC) are what allow JT65 to work so well at very low signal levels (as low as -25 dB S/S+N ratio). The bandwidth of a JT65 transmission is 175 Hz. A typical QSO requires 5-6 message exchanges, each lasting 1 minute for a total QSO time of 5-6 minutes.

JT65-HF Links For Downloads/Setup Manuals, etc:

1) JT65-HF Setup Manual & Technical Details - 
          https://dl.dropboxusercontent.com/u/26171574/JT65/jt65-hf-setup.pdf

2) JT65-HF Install/Setup program -
          https://dl.dropboxusercontent.com/u/26171574/JT65/setup-JT65-HF-1093.exe

3) Link to download the TimeSync 2.0 Program (which you will need for both JT65 and JT9):
          http://ravib.com/timesync/

4) Excellent article on communicating with JT65 by W6DTW:
         https://dl.dropboxusercontent.com/u/26171574/JT65/2010_10_JT65A-Part-1_Compressed.pdf

Typical Band/Frequencies (Xcvr Dial Setting - USB) used for JT65-HF:

160 Meters  -  1.838 MHz
	 80 Meters   -  3.576 MHz
	 40 Meters   -  7.076 MHz
	 30 Meters   - 10.139 MHz
	 20 Meters   - 14.076 MHz
	 17 Meters   - 18.102 MHz
	 15 Meters   - 21.076 MHz
	 12 Meters   - 24.917 MHz
	 10 Meters   - 28.076 MHz

JT9 Mode General Description – This mode is a relatively new mode contained in the WSJT-X version of WSJT, developed by the WSJT Project (headed by Joe Taylor - K1JT). JT9 is similar to JT65 and also uses a tightly structured protocol that sends a very limited set of structured messages or a small ‘free text’ message type limited to a maximum of 13 characters.

JT9 is currently optimized for the 1.8 MHz, 472 kHz, 137 kHz, and HF bands. It is about 2 dB more sensitive than JT65 while using less than 10% of the bandwidth.

JT9 offers five choices for the duration of timed T/R sequences (sub-modes). These sub-modes are JT9-1, JT9-2, JT9-5, JT9-10, and JT9-30 which use 1, 2, 5, 10, and 30 minute transmission periods respectively. Using longer transmissions trades reduced throughput for smaller bandwidth and increased sensitivity. The slowest sub-mode, JT9-30, has total bandwidth of only 0.4 Hz and operates at signal-to-noise ratios as low as -42 dB measured in the standard 2.5 kHz reference bandwidth! JT9-1 is the most commonly used sub-mode unless you really need the additional sensitivity of a slower mode. Probably 99% of all JT-9 contacts made today use the JT9-1 sub-mode.

Like JT65, JT9 is not a free form data transmission protocol, but each transmitted ‘frame’ is of a fixed length, starting and ending at a predefined time. Unlike JT65, JT9 offers 5 sub-modes using different length ‘frames’. JT9’s unique structure, timing, modulation method, and heavy Forward Error Correction (FEC) are what allow JT9 to work even better than JT65 at very low signal levels.

The JT9 mode is only available from within a special software program named WSJT-X.  Early versions of WSJT-X did not contain JT65 optimized for use on HF. This is probably what inspired W6CQZ to create the JT65-HF software package. The very latest version of WSJT-X software will operate in either JT65 or JT9 modes (or both simultaneously!). Whether or not the WSJT-X version of JT65 will supersede JT65-HF remains to be seen. Personally, I still prefer to use the JT65-HF software as it seems to be more user friendly, but you will need to decide for yourself which software to use.  The nice thing is that you can have both software programs installed on your computer. You just can’t run them both at the same time.

JT9 Links For Downloads/Setup Manuals, etc:

1) WSJT-X Page For Downloading/Manuals/Etc. -
          http://www.physics.princeton.edu/pulsar/K1JT/wsjtx.html 

     2) Latest Version of WSJT-X install (Contains Both JT9 & JT65) -
         https://dl.dropboxusercontent.com/u/26171574/WSJT- X%20Beta%20Version%201.2.1%20r3590/WSJTX_121r3590.exe

3) Latest WSJT-X Users Guide Ver 1.1 (PDF) -
        https://dl.dropboxusercontent.com/u/26171574/WSJT-X%20Beta%20Version%201.2.1%20r3590/WSJT-X_Users_Guide_v1.1.pdf    

     4) Latest Change Log for WSJT-X -
         https://dl.dropboxusercontent.com/u/26171574/WSJT-X%20Beta%20Version%201.2.1%20r3590/wsjtx_changelog.txt

Detailed Properties For JT9 Sub-Modes:

Sub-Mode            Bandwidth      Sensitivity     Total QSO Time

JT9-1			15.6 Hz	-27 dB	  	  6 Minutes
JT9-2		 	7.0 Hz		-30 dB		 12 Minutes
JT9-5		 	2.6 Hz		-34 dB	 	 30 Minutes
JT9-10	 	1.3 Hz		-37 dB	 	 60 Minutes
JT9-30 	 	0.4 Hz		-42 dB		180 Minutes

Typical Band/Frequencies (Xcvr Dial Setting - USB) used for JT9:

160 Meters  -   1.840 MHz
	 80 Meters   -   3.578 MHz
	 40 Meters   -   7.078 MHz
	 30 Meters   - 10.141 MHz
	 20 Meters  -  14.078 MHz
	 17 Meters  -  18.104 MHz
	 15 Meters  -  21.078 MHz
	 12 Meters  -  24.919 MHz
	 10 Meters  -  28.078 MHz

I’m certain that by now, all this technical chatter has boggled your mind, at least a little bit. All I can advise at this point is for you to download the manuals (using the links given earlier), print them out, and then start studying. Also, don’t be afraid to install the software, set it up, and start using it on the air.  As the little boy sitting on a potty said in an earlier article, “Nothing worthwhile is accomplished without effort!”  Trust me - learning how to operate using these two new modes will pay you back “in spades”! Besides, if you need help or don’t understand something, I will be pleased to help.  All you need do is contact me at 303-799-3658 or w0nac@comcast.net. If needed, we can even set up a help session (using Teamviewer) where I can control your computer remotely and help sort things out.

By now, you are probably asking yourself “How could any mode that takes 5-6 minutes for a single QSO be useful in county hunting?” The answer is that both JT65 and JT9 are far too slow to use for “putting out” counties while mobile. However, they can be very productive while doing what I call “trolling”. Trolling is where you are just calling CQ or answering other’s CQs using JT65 or JT9. You can “troll” while you are operating fixed or mobile. In theory, you can complete 10-12 QSOs per hour and if all of these contacts are new, you could “bag” 10-12 new counties during this hour.  Compare this to working mobile stations on the net. Here, assuming 10 minute runs, you can make a maximum of only 6 QSOs per hour. To be fair, logging fellow county hunters will probably yield more “goodies” (Stars, MG, MP, MD, etc.) than random fixed stations will, but “trolling”, as described earlier, can be very productive toward new counties for USACA, call prefixes, new modes for the 5 – Mode Award and new counties for the USA – Digital Award.

All the above is theoretical and there are many factors that will cut back on the “goodies” you can log for both scenarios. However, if you look on “trolling” as a supplement to working mobiles and NOT as a replacement, “trolling” can be very useful and a lot of fun at the same time!

December Activities:

Digital activities during December remain low. The only digital stations spotted on W6RK were:
11/30  W0NAC/N0LXJ   Clear Creek/Gilpin, CO	PSK-63, RTTY-45, MFSK-32, DOMINO-22
12/06  KA4RRU              Fauquier, VA			PSK-63
12/27  NA8W	 	       Holmes, OH			PSK-63

There were undoubtedly many other digital stations out there, but they were not county hunters and were not spotted on W6RK.

The updated Table 3 is given below:

How To Get Your 5 – Mode Statistics Updated:

Several past Digital Happenings articles have given detailed instructions on a new and easy way to update your digital statistics for Table 3 by using a small program called “Digital Report Utility”.  This new program will create a report for you showing your progress for both the USA – Digital and the 5 – Mode Awards and email it to W0NAC.  Instead of repeating these instructions every month I have included them in a Microsoft Word document that you can download by clicking on the following link: https://dl.dropboxusercontent.com/u/26171574/Happenings/5%20Mode%20Update.doc

Possible topics for the coming months include (in no particular order):

“How To Log Your Digital Contacts In Logger”
“Macros – Revisited”
“New Digital Awards?”
“Digital Software Installation – Revisited”
“Award Difficulty Index”

Come on Guys and Gals! I still really need (and welcome) your suggestions on topics for future articles!  Or, if you have something you wish to contribute I will certainly try to include it.  Short stories describing your experiences/joys/frustrations/etc. in getting started in digital ops would be especially welcome.  Summaries of your digital trips would also be good.  I look forward to hearing from you. Don’t be shy!

Please email me with your comments/suggestions at w0nac@comcast.net and don’t forget to send your updated status for the USA – Digital and 5 - Mode Awards  so I can update the Active Digital County Hunters Award Status list again next month.

73’s and we hope to see you on our waterfall again real soon!

Matt – W0NAC